Size of Space and Occupant Design Criteria: All boards shall use the Size of Space and Occupant Design Criteria contained in this section for planning new and evaluating existing educational, auxiliary, and ancillary facilities.

Procedures for Use of Size of Space and Occupant Design Criteria:

(1)	Boards, including the School Boards, Community College Boards of Trustees, University Boards of Trustees, Board of Governors, and public broadcasting stations may use the size of space and occupant design criteria contained in this section to develop educational specifications and user requirements for use by designers in the development of phase I, II, and III documents.

(2)	In all the Size of Space and Occupant Design Criteria tables, the square footage is provided for the convenience of selecting a nominal size.

In Table (A) for Public Schools and Vocational-Technical Schools, the indicators for Grade Level are: "N" for nursery, "P" for preschool, "K" for kindergarten, "1-12" for grades one through twelve, and "PS" for postsecondary vocational programs. Instructional spaces that contain student stations are marked with an asterisk (*).

In Table (B) for Community Colleges, the Information Classification Structure (ICS) Code identifies the type of program or function associated with a given set of spaces. The same ICS Codes are used in the Room Inventory of the Community College Facilities Inventory.

In Table (C) for State Universities, the Classification of Instructional Programs (CIP) Code identifies the particular academic discipline associated with various classroom, teaching laboratory, and research laboratory spaces.

(3)	When using the Size of Space and Occupant Design Criteria tables to calculate net square footage, the following procedure may be used. Select the desired facility space. For some facility spaces, the recommended range of sizes is listed to the right.

For most non-core curricula classroom facility spaces, the recommended size depends on the number of occupants, or other kind of unit, the facility space needs to house. In these cases, the number of occupants, or other unit, is multiplied by the square feet per occupant or unit to get the size of the main space. Core-curricula classrooms are assigned student stations based on the type classroom.

Related spaces are suggested for many facility spaces. They are indicated by FISH (Florida Inventory of School Houses) codes for Public Schools and Vocational-Technical Schools, and by alphanumeric codes for Community Colleges and State Universities. The codes are shown in the rightmost column. They are used to look up the names and sizes of the related spaces, which are found at the end of Table (A) for Public Schools and Vocational-Technical Schools and in the separate Table (D) Related Spaces for Community Colleges and State Universities. The square footages for the related spaces are added to the size of the main space to get the total net square footage for the program.

(4)	Once program net square footages are determined, other building space may be estimated as follows:

The aggregate amount of program net square footage may be increased up to six (6) percent for interior enclosed space needed for electrical, mechanical, and HVAC equipment. The result is total net square footage for the building.

The square footage for groupings of instructional spaces without fixed seating and without floor-to-

ceiling walls may be enlarged by four (4) additional square feet per student for circulation space. This additional circulation space should be excluded from the building net square footage amount used to figure the net-to-gross difference explained below.

The total building net square footage may be supplemented for general circulation, interior and exterior walls, open malls, and roof overhangs. The additional space is the net-to-gross square footage difference for the building. The recommended amounts are as follows:

(a)	Elementary school (grades N-6): twenty-seven (27) percent of building net square footage.

(b)	Middle school (grades 6-9): thirty-two (32) percent of building net square footage.

(c)	High school (grades 9-12): thirty-four (34) percent of building net square footage.

(d)	Community college, state university, ancillary, and public broadcasting: thirty-four (34) percent of building net square footage.

Refer to the Facility Space Chart (OEF Form 208A, which is a supplement to OEF Form 208, Letter of Transmittal) following these tables, for methods of measuring and calculating net square footage, net-to-gross difference square footage, and gross square footage.

(5)	Five Size of Space and Occupant Design Criteria tables are provided.

Table (A) Public School, Vocational-Technical, and Related Spaces for Public Schools and Vocational-Technical Schools.

Table (B) Community Colleges.

Table (C) State Universities.

Table (D) Related Spaces for Community Colleges and State Universities.

Table (E) Public Broadcasting Stations.

(6)	For exiting requirements, refer to Florida Building Code and NFPA 101 (National Fire Protection Association).

- District school boards have the option to continue an interlocal agreement that was developed utilizing data from the Florida Inventory of School Houses (FISH) as of October 1, 2004. After the expiration date, a new interlocal agreement will be required based on up-to-date FISH data.

(8)  For each change in any educational facilities space that results in an increase or decrease in net square footage of the space, student stations, the actual design of a space, or the condition of a space, school districts shall report the change as follows:

(a)	The facilities inventory shall be corrected by submitting on-line transactions through the Educational Facilities Information System (EFIS).

(b)	A district’s facilities inventory shall be corrected when new additions or remodeling occurs, during a validation study, or other event that causes or results in a change in square footage, student stations, design of a facilities space, or the condition of a facilities space.

1. New construction shall be added to the facilities inventory when a construction contract is issued.

2. Areas that are scheduled to undergo remodeling shall be updated in the inventory when a construction contract is issued.

3. When a remodeling project is conducted by district staff, the inventory shall be updated when the project is substantially completed.

(c)	Prior to October 1 of each year, each district shall review the Florida Inventory of School Houses (FISH) and shall certify to the Office of Educational Facilities that the inventory is current and accurate.

1. GENERAL EDUCATION SPACE (N-12)

a. Core-curricula

001	PK-3	Primary	*18	1	  49	808, 811, 813, 814

002	4-8	Intermediate/Middle	*22	1	39	808, 811, 815, 816

003	9-12	Senior High	*25	1	32	808

010	PK-3	Primary-Skills Lab	*18	1	49	808, 813, 814

(1 per each 400 student stations without FISH

capacity, additional rooms will have capacity)

011	4-8	Intermediate/Middle-Skills Lab	*22	1	 39	808, 815, 816

012	9-12	Senior High-Skills Lab	*25	1	32	808

020	4-8	Intermediate/Middle-Science Demonstration	*22	1	37	808, 812

021	4-8	Intermediate/Middle-Science Lab	*22	1	51	808, 812

022	9-12	Senior High-Science Demonstration	*25	1	37	808, 812

023	9-12	Senior High-Science Lab	*25	1	51	808, 812

030	PK-3	Primary-Open Plan	*36, 54, 72	2, 3, 4	38	808, 813, 814

031	4-8	Intermediate/Middle-Open Plan	*44, 66, 88	2, 3, 4	32	808, 815, 816

032 	9-12	Senior High-Open Plan	*50, 75, 100	2, 3, 4	27	808

060	N-PK	ESE Pre-K	*5	1	95	808, 813, 817

061	PK-12	ESE Part-Time	*15	1	65	808, 813, 815, 816

062	PK-12	ESE Full-Time	*10	1	95	808, 813, 815, 816,

817

063	PK-12	ESE Vocational	*12	1	95	808, 815, 816

NOTE: All fund sources that require an approved survey recommendation and compliance with the cost per student station as specified in F.S. 1013.64(6)(b)1, must not exceed the specified cost per student station based on the maximum allowable NSF per student station for the total project. The cost per student station maximum does not apply to projects with a fund source that is not regulated by an approved survey recommendation and the student station cost maximums established in F.S. 1013.64(6)(b)1.

b. Non-Core-Curricula Instructional Support

040	PK-12	Resource Room	*10	1	29	808

(1 per each 150 stations in elementary schools and

1 per each 250 stations in middle/high schools

without FISH capacity; additional resource rooms

will have capacity)

050	PK-5	Art - Elementary	*22	1	1000	808, 812

(1 per elementary school without FISH

capacity; additional rooms will have capacity)

051	4-8	Art - Intermediate/Middle	*30	1	42	803, 805, 808, 812

052	9-12	Art - Senior High	*30	1	53	803, 805, 808, 812

064	PK-12	ESE PT/OT	5	1	95	808, 813, 817

065	PK-12	ESE Resource	*4	1	95	808, 813

(1 per each 500 stations without FISH capacity;

additional ESE resource rooms will have capacity)

066	PK-12	ESE Supplemental Instruction	2	1	50	808

067	PK-12	ESE Observation Booth	  		150

068	PK-12	ESE Time Out	 		40

069	PK-12	ESE Audiology Lab			250	808

070	PK-12 	Itinerant	4	1	50	808

071	PK-12	Therapy Pool	one		1000	808, 818(2)

(Profound centers only)

c. Music

055	PK-5	Music	*22	1	1000	806, 808, 831

(1 per elementary school without FISH

capacity; additional rooms will have capacity)

075	6-12	Vocal Music Classroom	*25	1	57	806, 808, 830, 831, 833, 836, 837

076	6-12	Band Classroom **	one	1	2000	806, 808, 830, 831, 832, 834, 835, 836, 837

077	6-12	Orchestra Classroom	*25	1	57	806, 808, 830, 831, 832, 836, 837

078	6-12	General Music Classroom	*25	1	37	808, 832

079	6-12	Guitar Laboratory	*25	1	37	808, 832

080	6-12	Piano Laboratory	*25	1	37	808

081	6-12	Recording Room	5		45

082	6-12	Instrument Repair	one		110

083#	6-12	Music Related Space (use for spaces not found

in design codes 830-837)

** Student stations are assigned to design code 076 for band classrooms as follows:

Total Satisfactory Student Stations

(Excluding gymnasiums,

and band classrooms) 	Assign Band Stations

240 or less 	30

241 - 820 	35

821 - 1080 	40

1081 - 1340 	45

1341 and above 	50

d. Physical Education

013	PK-5	Physical Education Storage	one		315

014	PK-5	PE Covered Play Area (one per school)	10% cap		36

090	6-12	Dressing Room - Male	5% cap		12

091	6-12	Dressing Room - Female	5% cap		12

092	6-12	Lockers - Male	5% cap		2

093	6-12	Lockers - Female	5% cap		2

094	6-12	Showers - Male	5% cap		2

095	6-12	Showers - Female	5% cap		2

815	6-12	Rest Room - Male	5% cap		2

816	6-12	Rest Room - Female	5% cap		2

096	6-12	Drying Area - Male	5% cap		2

097	6-12	Drying Area - Female	5% cap		2

098	6-12	Storage	5% cap		9

099	6-12	Teachers Shower - Male	one		22

100	6-12	Teachers Shower - Female	one		22

110	6-12	Multipurpose/Instruction	one		1050

111	6-9	Gymnasium Floor ***	one	1	5800

112	9-12	Gymnasium Floor ***	one	1	6500

113	6-12	Gymnasium Seating	10% cap		32

114	6-12	Laundry/Towel Distribution	5% cap		2

115	6-12	First Aid	5% cap		2

116	6-12	Training Room (with whirlpool)	one		250

117	6-12	Weight Room	one		1000

118	6-12	Wrestling Room	one		1680

119	6-12	Gymnastics/Dance	one		1050

120	6-12	Gymnasium Storage	 5% cap		3

121#	6-12	Other Physical Education Space

(use for spaces not found in design codes 800-827)

*** Student stations are assigned to design codes 111 and 112 for gymnasiums as follows:

Total Satisfactory Student Stations

(Excluding gymnasiums,

and band classrooms) 	Assign PE Stations

240 or less 	40

241 - 820 	60

821 - 1080 	80

1081 - 1340 	120

1341 and above 	160

2. VOCATIONAL-TECHNICAL SPACE (6-PS)

a. Agricultural Education

200	6-9	Orientation & Exploration Laboratory	*22	1	40	808, 812, 840, 841

201	9-12	Practical Experience Laboratory	*25	1	50	806, 810, 847, 848, 850,840, 841

202	9-PS	Small Education Laboratory	*20	1	55	806, 810, 818(2), 840, 841, 847, 848, 850

203	9-PS	Medium Education Laboratory	*20	1	80	806, 810, 818(2), 840, 841, 847, 848, 851

204	9-PS	Large Education Laboratory	*20	1	128	806, 810, 818(2), 840, 841, 847, 848, 851

b. Business Education

210	6-9	Orientation & Exploration Laboratory	*22	1	55	808

211	9-12	Practical Experience Laboratory	*25	1	62	808

212	9-PS	Education Laboratory	*20	1	73	808

c. Distributive and Diversified Education

220	6-9	Orientation & Exploration Laboratory	*22	1	40	808

221	9-12	Practical Experience Laboratory	*25	1	42	808

222	9-PS	Small Education Laboratory	*20	1	55	812, 840

223	9-PS	Medium Education Laboratory	*20	1	100	808, 812, 840

224	9-PS	Large Education Laboratory	*20	1	200	810, 812, 840

d. Family and Consumer Sciences

230	6-9	Orientation & Exploration Laboratory	*22	1	70	808, 812, 842, 843, 852

231	9-12	Practical Experience Laboratory	*25	1	64	808, 843, 852

232	9-PS	Small Education Laboratory	*20	1	55	812, 852

233	9-PS	Medium Education Laboratory	*20	1	69	808, 842, 843, 852

234	9-PS	Large Education Laboratory	*25	1	90	812, 842, 843, 852

e. Technology Education

240	6-9	Orientation & Exploration Laboratory	*22	1	95	808, 849, 851, 852

241	9-12	Small Education Laboratory	*25	1	65	808, 852

242	9-12	Medium Education Laboratory	*25	1	95	810, 852

243	9-12	Large Education Laboratory	*25	1	135	808, 810, 849, 851, 852

f. Industrial Education

244	9-PS	Small Education Laboratory	*20	1	55	808, 840

245	9-PS	Medium Education Laboratory	*20	1	90	808, 810, 840, 849, 850

246	9-PS	Large Education Laboratory	*20	1	200	808, 810, 840, 847, 849, 850

g. Health Occupations Education

250	6-9	Orientation & Exploration Laboratory	*22	1	46	808

251	9-12	Practical Experience Laboratory	*25	1	56	808

252	9-PS	Small Education Laboratory	*20	1	60	804, 808, 812, 840

253	9-PS	Medium Education Laboratory	*20	1	110	804, 806, 808, 810, 812, 840, 849

254	9-PS	Large Education Laboratory	*20	1	165	804, 806, 810, 818, 840, 849

h. Public Service Education

260	6-9	Orientation & Exploration Laboratory	*22	1	46	808, 810

261	9-12	Practical Experience Laboratory	*25	1	55	808

262	9-PS	Small Education Laboratory	*20	1	40	808

263	9-PS	Medium Education Laboratory	*20	1	65	810, 840

264	9-PS	Large Education Laboratory	*20	1	98	810, 840

i. Vocational Resource Space

270	9-PS	Work Evaluation Laboratory	*15	1	74	810, 853

(one per school)

271	9-PS 	VPI Vocational Preparatory Instruction	*15	1	47	802,808,840,846,853

(one per school)

272#	9-PS 	Vocational Laboratory Support

(use for spaces not found in design codes 840-870)

Note 1:  Related and select spaces may be added or deleted based on the unique vocational program needs as supported by enrollment, projections, COFTE, and other data. See AVocational Recommended Spaces@ crosswalk for individual program recommendations provided by the Office of Workforce and Economic Development.

Note 2:  As per Section 1013.31, F.S., the Office of Workforce and Economic Development shall establish and transmit to the Office documentation of the need for programs.

Capacity: The number of students that may be housed in a facility at any given time based on a utilization percentage of the total number of existing satisfactory student stations:

Utilization		Satisfactory

Factor		   Student

Type School		Percentage	   Stations

Elementary 		100% 		All

Middle & Junior High 		90% 		All

Senior High 		70% 	300 or less

75% 	301 - 600

80% 	601 - 900

85% 	901 - 1200

90% 	1201 - 1500

95% 	1501 - or more

Combination Schools 		90% 		All

Exceptional Student Centers 		100% 		All

Alternative Education Centers 		100% 		All

Designated Area Vocational Centers		120%		All

Designated Adult Centers  		150%		All

Note 3: Adult and Vocational Centers have increased utilization factors because of specialized day, evening, and weekend use of facilities.

3. AUXILIARY SPACE (N-PS)

a. Administration/Student Services

300	N-PS	Principal's/Director's Office	each	250

301	N-PS	Assistant Principal/Media/Administrative/	each	175

Guidance Office

302	N-PS	Bookkeeping Office	each	125

303	N-PS	Secretarial Space	each	158

304	N-PS	General Administrative Reception Area	5% cap	17

305	N-PS	Production Workroom	5% cap	8

306	N-PS	Conference Room	5% cap	14

307	N-PS	Clinic	5% cap	 6

308	N-PS	Administrative Storage	5% cap	10

309	N-PS	Records Vault/Student Records	5% cap	6

310	N-PS	School Store	5% cap	2

311	N-PS	Student Activities Area	5% cap	10

312	N-PS	Computer Area	5% cap	3

313	N-PS	Careers Room	5% cap	6

314	N-PS	Itinerant Office (one per each 400 stations)	each	125

315	N-PS	Teacher Planning Office	10% cap	20

316	N-PS	Teacher Lounge/Dining	10% cap	4

317#	N-PS	 General Administrative Space

(use for spaces not found in design codes 800-827

b. Custodial

330	N-PS	Custodial Receiving	10% cap	15

331#	N-PS	Service Closets

332#	N-PS	Work Area

333	N-PS	Flammable Storage	one	155

334	N-PS	Equipment Storage	one	500

c. Food Service

340	N-PS	Dining Area	10% cap	40

341	N-PS	Kitchen and Serving Area	10% cap	44

342#	N-PS	Kitchen Dry Storage Area

343#	N-PS	Kitchen Office

344#	N-PS	Kitchen Garbage Wash Area

345#	N-PS	Kitchen Non-food Storage Area

346#	N-PS	Kitchen Food Preparation Area

347#	N-PS	Kitchen Dish Washing Area

348#	N-PS	Satellite Kitchen

349	N-PS	Chair Storage   	5% cap	4

350#	N-PS	Other Food Service

(use for spaces not found in design codes 800-827)

351	6-12	Covered Patio	10% cap	36

d. Auditorium (cannot be included with multipurpose room)

360	6-PS	Auditorium Seating	10% cap	30

e. Multipurpose (cannot be included with auditorium)

361	N-PS	Multipurpose Room	10% cap	31

362	N-PS	Chair Storage	10% cap	2

f. Stage

363	N-PS	Stage attached to auditorium, multi-	one	990

purpose, gym, or dining

364	N-PS	Storage	10% cap	5

365	N-PS	Dressing - Male	  5% cap	5

366	N-PS	Dressing - Female 	  5% cap	5

367	N-PS	Control Booth/Projection Room	one	100

g. Textbook Storage

368	N-PS	Textbook Storage Area	  5% cap	7

h. Student Storage

369	6-PS	Student Personal Storage         	10% cap	5

i. Public Use

(With Auditorium and/or Gymnasium per school)

370	6-PS	Lobby	  5% cap	10

371	6-PS	Concessions	one	200

372	6-PS	Ticket Booth	one	30

j. School Media Center

380	P-PS	Reading Room/Stacks	10% cap	37

381	P-PS	Technical Processing Area	10% cap	4

382	P-PS	Production & Professional Library	10% cap	4

383	P-PS	AV Storage Area	10% cap	6

384	P-PS	Periodical Storage Area	10% cap	2

385	P-PS	Closed Circuit TV (Production,	10% cap	7

Distribution, and Control)

386	P-PS 	Closed Circuit Storage Area	10% cap	5

387	P-PS	Media Production Laboratory	10% cap	5

388	P-PS	Copying Room	10% cap	2

389	P-PS	Small Group Room (view & preview)	  5% cap	2

390	P-PS 	Group Projects and Instruction	10% cap	5

391	P-PS	Media Maintenance and Repair	  5% cap	2

4. ANCILLARY SPACE (DISTRICT)

Total Ancillary Allocation = Survey Projected COFTE x NSF Factor

NSF

COFTE	Factor

0 -  10,000	6.00

10,001 -  20,000	5.75

20.000 -  30,000	5.50

30,001 -  50,000	5.25

50,001 - 100,000	5.00

100,001 - 200,000	4.75

200,001 - 600,000	4.50

a. Ancillary Administrative Support (38%)

NSF allocated for ancillary administrative support is to be distributed by the district among design codes 400-415 and 417-428.

400		Superintendent		200

401		Conference Room		100

402		Superintendent's Secretary

403		Ancillary Secretarial/Clerical Offices

404		Ancillary Reception Area		 100

405		Vault	 	100

406		Assistant Superintendent		180

407		Ancillary Administrative Offices	 	100

408		Business Operations

409		Terminal Storage Area (Business Operations)

410		School Plant Planning

411		Word Processing Center

412		Personnel Services

413		Central Reproduction and Copy

414		Central Administrative Supply

415		Mail Room

417		Central Security

418		Ancillary Administrative Storage

419		Ancillary Flammable Storage

420		Board Meeting Room		500

421		Ancillary Staff Lounge		200

422		Main Lobby and Switchboard

424		Director's Office

425		Assistant Director's Office

426		General Office

427		Staff Development/Instructional

428#		Other Ancillary Administrative Support

b. Ancillary Custodial Services (2%)

NSF allocated for ancillary custodial services is to be distributed by the district for design code 416.

416		Custodial Services

c. Ancillary Computer/Data Center (2%)

NSF allocated for ancillary computer/data centers is to be distributed by the district among design codes 500-506.

500		Programmer Room

501		Data Processing Technical Area

502		Data Processing Equipment

503		Computer Room (Raised Floor)

504		Off-Line Equipment Room

505		Ancillary Computer Storage

506#		Other Central Equipment Support

d. Ancillary Support Facilities (50%)

NSF allocated for ancillary support facilities is to be distributed by the district among design codes 510-594.

510		Warehouse

515		Central Kitchen

520		Carpentry Shop

525		Glazing Shop

530		Masonry Shop

535		Small Engine Shop

540		Electronics Shop

545		Electrical Shop

550		Machine Shop

555		Plumbing Shop

560		Paint Shop

565		Welding Shop

570		Air Conditioning

575		Carpet Shop

580		Locksmith Shop

585		Garage Parts room

586		Machine Shop

587		Glass/Upholstery Shop

588		Body Shop

589		Paint/Flammable Storage

590		Paint Bay		800

591		Tire Storage & Mounting

592		Work Bay		80

593		Drivers' Classroom		400

594		Ancillary Support Storage

e. Ancillary Media Services (8%)

NSF allocated for ancillary media services is to be distributed by the district among design codes 600-612.

600		Library Warehouse/Stacks

601		Reference

602		Professional Library

603		Periodical/Journal Services

604		Central Media Processing

605		Audio Visual Equipment

606		Closed Circuit TV Laboratory

607		Closed Circuit Support

608		Media Production Laboratory

609		Media Copying Room

610		Media Maintenance/Repair

611		Ancillary Media Storage

612#		Other Ancillary Media Space

5. SPECIAL USE DESIGN CODES

700#		Inside Circulation Area

701#		Covered Walkway

702#		Mechanical Room

703#		Electrical Room

704#	K-12 	In School Suspension or Detention Room	*20	1	30	808, 815, 816

705#		Museum/Gallery/Art Display Room

707#		Telephone Equipment Room

708#	9-12	J.R.O.T.C.	*25	1	42	800, 801, 802, 808

6. RELATED SPACES

a. Combination and General Use Related Spaces

800		Arms Room  		 	150	708

801		Firing Range (indoor)		 	2400	708

802		Conference (instructional) 		 	225	708, 271

803		Darkroom    		 	100	051, 052

804		Dispensary      		 	135	252, 253, 254

805		Kiln 			60	051, 052

806		Reference			100	055, 075, 076, 077, 201, 202, 203, 204, 253, 254

808		Storage			100	001, 002, 003, 010, 011, 012, 020, 021, 022, 023, 030, 031, 032, 040, 050, 051, 052, 055, 060, 061, 062, 063, 064, 065, 066, 069, 070, 071, 075, 076, 077, 078, 079, 080, 200, 210, 211, 212, 220, 221, 223, 230, 231, 233, 240, 241, 243, 244, 245, 246, 250, 251, 252, 253, 260, 261, 262, 271, 704, 708

810		Storage, Material (large)			395	201, 202, 203, 204, 224, 242, 243, 245, 246, 253, 254, 260, 263, 264, 270

811		Storage, Outside    			50	001, 002

812		Storage, Project (small)			150	020, 021, 022, 023, 050, 051, 052, 200, 222, 223, 224, 230, 232, 234, 252, 253

813		Storage, Student (N-3, ESE, & Vocational Education)			40	001, 010, 030, 060, 061, 062, 064, 065

814		Student Rest Rooms - Male/Female (PreK-3)			60	001, 010, 030

815		Student Rest Rooms - Male (4-12)	5% cap		15

816		Student Rest Rooms - Female (4-12) 	5% cap		15

817		Student Rest Room and Bath (ESE)			110	060, 062, 064

818		Lockers, Rest Rooms, and Showers (ESE & 			225	071, 202, 203, 204, 254

Vocational Education)

819		Rest Rooms, Staff - Male	5% cap		4

820		Rest Rooms, Staff - Female	5% cap		4

821#		Rest Rooms, Staff - Male/Female

822		Public Rest Rooms - Male	5% cap		2

823		Public Rest Rooms - Female	5% cap		2

824		Rest Rooms, Ancillary - Male	5% COFTE		2

825		Rest Rooms, Ancillary - Female	5% COFTE		2

826#		Elevators, Freight/Passengers

827#		Elevators (Passenger/Handicapped)

b. Music Related Spaces

830		Ensemble  			300	075, 076, 077

831		Practice, Music, one per 40 students			70	055, 075, 076, 077

832		Storage, Instrument 			600	076, 077, 078, 079

833		Storage, Robe  			150	075

834		Storage, Uniform    			300	076

835		Studio    			180	076

836		Sheet Music Storage			150	075, 076, 077

837		Storage, Large Equipment			400	075, 076, 077

c. Vocational Related Spaces

840		Classroom for Related Instruction	*20	1	34	200, 201, 202, 203, 204, 222, 223, 224, 244, 245, 246, 252,

253, 254, 263, 264, 271

(Stations are assigned for any space other than approved

classrooms associated with vocational laboratory.)

841		Greenhouse     			800	200, 201, 202, 203, 204

842		Kitchen (Family and Consumer Sciences)  		   	125	230, 233, 234

843		Laundry (Family and Consumer Sciences)  			50	230, 231, 233, 234

846		Reception (instructional) 		  	90	271

847		Storage, Flammable 		 	125	201, 202, 203, 204, 246

848		Storage, Machinery 			1100	201, 202, 203, 204

849		Storage, Project (large)	  		310	240, 243, 245, 246, 253, 254

850		Storage, Tool (small) 			195	201, 202, 245, 246

851		Storage, Tool (large) 			310	203, 204, 240, 243

853		Testing 			250	270, 271

852		Technology Resource Center			800	230, 231, 232, 233, 234, 240, 241, 242, 243

d. Vocational Select Spaces

807		Storage, Equipment			315

844		Multipurpose Laboratory 			1200

(Family and Consumer Sciences)

845		Observation (Family and Consumer Sciences)			50

854		Vocational Darkroom			225

861		Animal Shelter			1000

862		Burn/Fire Maze Instruction			1100

863		Fitting Room			50

864		Isolation Room			45

865		Radio Control Room			100

866		Radio/Studio(2)			900

867		TV Control Room (2)			600

868		TV Studio (2)			1100

869		X-Ray			135

870		Test Cell			150